051005T4ICT

ICT Level 5

IT/OS/ICT/CR/01/5/A

Perform Computer Networking

Nov/Dec 2022

THE KENYA NATIONAL EXAMINATIONS COUNCIL

PRACTICAL ASSESSMENT

INSTRUCTIONS TO CANDIDATES:

- In this practical assessment, you are required to perform the following tasks:

TASK 1: Perform crossover cable Termination

TASK 2: Design a peer-to-peer network

TASK 3: Set up a peer to peer network

TASK 4: Test the network

TASK 5: Share files over the network

- You have Ten minutes to read through the instructions and check availability of resources for the practical.

- You are allocated 1 hour 30 minutes to perform all the tasks.

- Each task has specific instructions.

- All tasks carry 60 marks.

- The assessor will record your performance at critical points using audio-visual means.

You will be provided with the following resources:

- Crimping tool

- Two working computers

- Cat5 or Cat 6 cable

- Rj45 connectors

- Cable tester

- Wire stripper

- A plain paper

TASK 1 									(20 marks)

Perform crossover cable Termination

- Prepare the cable for termination

- Make one end to T568A color coding, place and crimp the connector.

- Make the other end to T568B color coding, place and crimp the connector.

- Test the cable

TASK 2

Design a peer-to-peer network						 (5 marks)

- Sketch on paper a network diagram for a peer-to-peer network connecting two computers directly.

- Label one computer with IP address 192.168.1.1 and the other computer with IP address 192.168.1.2.

- Use labels to indicate connecting media.

TASK 3

Set up a peer to peer network							(14 marks)

- Identify all the requirements needed for  a peer to peer network set up

- Set the name of the first computer to CDACC1 and the second computer to CDACC2

- Set  physical connectivity using the crossover cable you made

- Configure IP settings. Use IP Address 192.168.1.1, Subnet Mask 255.255.255.0 for the first computer and IP Address 192.168.1.2, Subnet Mask 255.255.255.0 for the second computer.

TASK 4

Test the network								(11 marks)

- Test the network connection using PING Utility

- Check Network Configuration using windows network and sharing center

TASK 5										(10 marks)

Share files over the network

- Create a folder named Kenya and Share it across the network.

- View and access the folder from the other computer in the network.