061006T4ICT

ICT TECHNICIAN LEVEL 6

IT/OS/ICT/CR/6/6

PERFORM COMPUTER REPAIR AND MAINTENANCE

Mar./ April 2023.

Time: 3 Hours

THE KENYA NATIONAL EXAMINATIONS COUNCIL

# PRACTICAL ASSESSMENT

# 3 hours

INSTRUCTIONS TO THE ASSESSOR

You are required to mark the practical as the candidate performs the task.

You are required to collect evidence i.e., video clips, photos at critical points.

Ensure that the candidate has a name tag and a registration code at the back and front.

Allocate 10 minutes for the candidate to identify and confirm the availability of the resources.

Candidate to perform the tasks as per the requirements of given.

The task carries 50 Marks.

OBSERVATION CHECKLIST

| Candidate’s name & Registration No. |  |  |  |
| --- | --- | --- | --- |
| Assessor’s name & Reg. code |  |  |  |
| Unit(s) of Competency | Perform computer repair & maintenance |  |  |
| Venue of Assessment |  |  |  |
| Date of assessment |  |  |  |
| (Indicate the marks available and marks obtained respectively. Award marks appropriately as guided for in the items for evaluation indicated. Give a brief comment where necessary) |  |  |  |
| Items to be evaluated: | Marks Allocated | Marks Obtained | Comments |
| - Grounded her/his self Award 1 mark or zero | 1 |  |  |
| - Prepared the Mainboard (motherboard). Award 2 marks or zero | 2 |  |  |
| - Mounted the CPU in the socket of the Mainboard. Award 2 marks or zero | 2 |  |  |
| - Connected the CPU cooler to the Mainboard. - Placed thermal compound to the CPU - Set the fan assembly on the CPU with mounting tabs aligned. - Pulled the locking rod down on the fan assembly to lock into place. - Connected the fan assembly's power connector to the motherboard. Award 1 mark for each or zero | 4 |  |  |
| - Attached the RAM (memory) modules in the corresponding slots. - Installed the module closest to the CPU - Set the RAM board in the socket - Locked tabs Award 1 mark for each | 3 |  |  |
| - Opened the case and mount the power supply. - Removed packaging materials - Removed side panels - Removed the cover for the optical drive. - Made note of the cables pre-installed in the case. - Aligned the mounting holes in the case and power supply - Inserted screws and tightened. Award 2 marks for each or zero | 6 |  |  |
| - Attached the Mainboard back plate to the case and checked the Mainboard mounting positions. - Installed the I/O bezel plate into the opening in the back of the case - Installed standoffs in the case Award 2 marks for each or zero | 4 |  |  |
| - Suitably positioned the Mainboard in the case. - Lowered the motherboard into the case and aligned it with the I/O bezel. - Installed the screws. Award 1 mark for each or zero | 2 |  |  |
| - Mounted the Hard disk and connect it to the power supply and the motherboard. - Mounted on the correct drive bay - Installed screws - Data cables connected to the disk and power cables to the motherboard. Award 1 marks for each or zero | 3 |  |  |
| - Connected the SATA connectors to the drives and the USB connectors and the case switches to the motherboard. - All drives connectors and case switches connected on the motherboard. Award 2 marks or zero | 2 |  |  |
| - Connected the 20 or 24 pin ATX connector and the 4-pin power supply control connector to the motherboard. Award 2 marks or zero | 2 |  |  |
| - Mounted the DVD-ROM drive. - Slid the drive into the drive bay until the screw holes are lined up and the front of the drive is flush with the front of the case - Installed the screws. Award 2 marks for each or zero | 4 |  |  |
| - After connecting the ATA cable to the device, hooked it up to the power supply. Award 2 marks or zero | 2 |  |  |
| - Closed the case and connected peripherals. - Placed the side cover back on - Secured the side panel with case screws - Connected peripheral devices including monitor, mouse, keyboard Award 1 mark for each or zero | 3 |  |  |
| - Installed the provided operating system. Award 10 marks for a working computer or zero | 10 |  |  |
| TOTAL | 50 |  |  |
| ASSESSMENT OUTCOME |  |  |  |
| The candidate was found to be: Competent Not yet competent (Please tick as appropriate) (The candidate is competent if s/he gets 50% or higher of the items of evaluation correct) |  |  |  |
| Feedback to candidate: |  |  |  |
| Feedback from candidate: |  |  |  |
| Candidate’s Signature Date ------------------------------ ----------------------------- |  |  |  |
| Assessor’s Signature Date ------------------------------ ----------------------------- |  |  |  |