# 061006T4ICT

ICT LEVEL 6

ICT/OS/IT/CR/1/6

PERFORM COMPUTER NETWORKING

Mar. /Apr. 2023

3 Hours

THE KENYA NATIONAL EXAMINATIONS COUNCIL

PRACTICAL ASSESSMENT

INSTRUCTIONS TO CANDIDATE

- This assessment requires the candidate to demonstrate competence against unit of competency: Perform Computer Networking.

- Time allocated: 3 Hours.

- In this assessment, you will be required to perform five (5) practical tasks.

- Write your name, registration code, date and sign in the practical assessment attendance register.

- You have 10 minutes to carefully read through the instructions and to collect the tools/resources required for the tasks.

- The assessor will record your performance at critical points using audio-visual means.

The following resources will be provided to the trainee:

- Three 2 to 3 ft. (0.6 to 0.9 meter) lengths of UTP cable, Category 5 or 5e or 6

- A minimum of 6 RJ-45 connectors (more may be needed if wiring mistakes occurs)

- An RJ-45 crimping tool

- Two computers with operating systems installed

- Wire cutter

- Wire stripper

- 1 Printer

- Standard long-nose pliers (both side-cutting and skinning)

- Cable tester

- Multimeter

- Wireshark

- Answer booklet

- Reliable Ethernet internet access.

- 1 Smart phone/Laptop. Desktop PC with Wi-Fi adapter.

In this assessment you are required to:

TASK 1: Design a physical and logical topology for the network. 	[15 Marks]

- In the answer booklet provided, create a physical topology of the network you intend to construct. The design should support both wired and wireless connections.

- Use 192.168.0.0/24 to create subnets that meet the host requirements for:

  - Staff: 100 hosts

  - Sales: 50 hosts

  - IT: 25 hosts

- Create an addressing table and document the IPv4 addresses that have been assigned in the table to the network devices.

TASK 2: Network the computers and the printer to enable them share resources over the network.									[60 Marks]

- Prepare adequate straight through ethernet cables for the network devices’ connection.

- Perform cable tests.

- Connect the devices as per the design in task 1 above.

- Configure the assigned IPv4 address, subnet mask, DNS address and default gateway settings on the computers using your addressing scheme.

- Configure the wireless router with a proper SSID, router IP address/subnet, DCHP server setting the starting IP address/maximum number of users, router. username/password and wireless security options.

TASK 3: Perform network test on the connected computers.		[15 Marks]

- Use the web browser on the computers to navigate to www.knec.go.ke

- All PCs should be able to ping all other the devices.

- Check the mac address tables of both computers and record their entries before and after the ping exercise.

- Test the internet speed of the network and save the test screen.

- Test connection to the wireless internet using the smart phone/laptop provided.

TASK 4: Print a file over the network printer.				[05 Marks]

- Test the functionality of the installed network printer by printing a test page.

TASK 5: Using appropriate software, perform network monitoring for the connected devices.									[05 Marks]

- Identify an appropriate tool that you can use to monitor the network performance.

- Capture screenshots of the network being monitored and save them in the ‘Pictures’ folder of one of the computers.

- Give an observation and a recommendation based on (b) above, in your answer booklet.