061006T4ICT

ICT Technician Level 6

IT/OS/ICT/CR/01/6

Perform Computer Networking

Nov. /Dec. 2022

THE KENYA NATIONAL EXAMINATIONS COUNCIL

PRACTICAL ASSESMENT

INSTRUCTIONS TO THE ASSESSOR

- You are required to mark the practical as the candidate performs the tasks.

- You are required to take video clips at critical points.

- Ensure the candidate has an identification tag pinned at the back and front near the shoulders showing Candidate’s name and registration code.

OBSERVATION CHECKLIST

| Candidate’s name & Registration No. |  |  |  |
| --- | --- | --- | --- |
| Assessor’s name & Reg. code |  |  |  |
| Unit(s) of Competency | Perform Computer Networking |  |  |
| Venue of Assessment |  |  |  |
| Date of assessment |  |  |  |
| (Award mark(s) appropriately as guided for in the items for evaluation indicated. Give a brief comment where necessary) |  |  |  |
| Items to be evaluated: | Marks Available | Marks Obtained | Comments |
| Task 1 |  |  |  |
| Perform straight through cable termination |  |  |  |
| - Created a physical topology of the network to be constructed. (Award 1 mark for each of the following in the diagram: Computer A, Computer B, Wireless router, Printer and award 1 mark if all cables are properly drawn) NB. The wireless router should be centrally placed in the diagram. | 05 |  |  |
| - Used 192.168.10.0/29 to create subnets that meet the host requirements for the network. (Award 1 mark for each of the following: Subnet mask: 255.255.255.255.248, Valid host IPs to be 4 per subnet e.g. (192.168.10.1 – 192.168.10.4), Subnets to be 32 in total) | 04 |  |  |
| - Created an addressing table and documented the IPv4 addresses to be assigned in the table. (Award 1 mark for each of the following: Valid host IP address, Valid subnet mask and Valid default gateway on each PC). The addresses should be in a table format. | 06 |  |  |
| - Prepared the cable for termination (Award 3 marks or zero for each of the following: Unsheathed the cable, cut cable wires to the appropriate length and checked the outer jacket and inner TP wires for nicks, arranged the wires correctly to meet T568A/B standards, crimped the cables properly and tested the cables) | 15 |  |  |
| - Configured the assigned IPv4 address, subnet mask, and default gateway settings on the computers using the addressing scheme. (Award 3 marks for correct settings on each computer and 9 marks for correct configuration of the router; check password, Ip addresses and DHCP settings on the router) | 15 |  |  |
| - Shared the ‘Documents’ folder on both computers to be accessible on the network. (Award 5 marks or zero) | 05 |  |  |
| - Configured the firewall settings on the two computers to deny any incoming connections to port 80 (Award 5 marks or zero for correct settings on each computer) | 10 |  |  |
| - Created a local domain named ‘Staff’ for the network and use it to share the network printer. (Award 2 ½ marks or zero for each computer with the correct domain settings. The assigned domain name should be ‘Staff’) | 05 |  |  |
| - Set up user account password on both computers to enhance network security (Award 2 ½ marks or zero for each computer with the correct password settings on the active account. A PIN or password may be used on the accounts) | 05 |  |  |
| - Run the ping command to test connectivity. (Award 5 marks if the test was successful or zero). The outcome should be documented in the booklet. | 05 |  |  |
| - Checked the mac address tables of both computers and recorded their entries before and after the ping exercise (Award 2 ½ marks for each correct status recorded for the PCs before and after or zero) | 05 |  |  |
| - Accessed the document, ‘Test Report’, from the second computer. (Award 2 marks saving the document in the right folder, 3 marks for accessing it over the network or zero if none is done) | 05 |  |  |
| - Enablde internet connection on the wireless router and tested the internet speed of the network. (Award 2 marks for successful connection to internet, 3 marks for testing and documenting the speed or zero) | 05 |  |  |
| - Tested the functionality of the installed network printer by printing ‘Test Report’. (Award 5 marks for a successful printout or zero) | 05 |  |  |
| - Performed network monitoring for the configured devices. (Award 5 marks for installing and running the Wireshark software to capture traffic or zero) | 05 |  |  |
| TOTAL | 100 |  |  |
| ASSESSMENT OUTCOME |  |  |  |
| The candidate was found to be: Competent Not yet competent (Please tick as appropriate) (The candidate is competent if s/he gets 70% or higher of the items of evaluation correct) |  |  |  |
| Feedback to candidate: |  |  |  |
| Feedback from candidate: |  |  |  |
| Candidate’s Signature Date ------------------------------ ----------------------------- |  |  |  |
| Assessor’s Signature Date ------------------------------ ----------------------------- |  |  |  |